Huairou Solar Observatory Station

1.INTRODUCTION

Huairou Solar Observing Station (HSOS) is located on an small island near the north bank of Huairou Reservoir, 60 kms north of the downtown of Beijing. The water surrounded the station extend about 2 kms to the east, south and west of the observing tower. The observing dome is on the top of a 31-meter-high tower. The HSOS's telescope is characterized by its movable dome, which can be shifted away from the telescope when the seeing is favorable. The surrounding water reduces the temperature fluctuation at the telescope height, which is favorable for high resolution solar observing. The annual observation days are about 300.

- Latitude: 40.4N

- Longitude: 116.6E

- Altitude: 62m

- Summer Temperature: Mean 26° C , Extreme Maximum >40° C

- Winter Temperature: Mean -4° C, Extreme Minimum -20° C

- Precipitation: 500mm

- Nearest Cities: Huairou 10kms, Beijing 60kms

- 2.INSTRUMENT

14-cm Full-disc & Local Hα  Telescope

A 14-cm telescope equipped with a tunable Hα filter, 2 CCD cameras for solar flare and other ephemeral phenomena observation.

Objective

Diameter 14cm

Filter

tunable H-alpha filer (-32Å ~ +32Å )
 bandwidth 0.5Å

Field Of View

Full-disc 34' x 34' or local region 10' x 8'

CCD

one for full-disc Hα  monochromatic image
another for local region image

Recently this system was equipped with a KD*P modulator and the chromosphere magnetic field is expected to be observed.

- 3.DATA

- The raw data format got by the 60cm Solar 3Channel Telescope is FIT, and the corresponding JPG picture is created by the related program from FIT. Only JPG datas are tranformed to the directory.

AS the replacement by the 20cm h-alpha telescope, 14cm alpha datas were not collected by the end of 2006.5.